prplHypervisor Deployment on PIC32MZ2048EF Platform

October 2016

Contents

1. Introduction	3

2. Prerequisites	3

3. Linux Host Packages Requirements	3

3.1 Remove ModemManager Software Package	3

4. Third Party Software and Tools	4

5. Hypervisor Deployment	4

5.1 Obtaining the prplHypervisor sources	5

5.2 Flashing the bootloader	5

5.3 Compiling the prplHypervisor	5

5.4 Uploading the firmware	7

5.5 Serial output	8

6. Hypervisor configuration	9

6.1 system group	9

6.1.1 Example of a valid system configuration	9

6.2 virtual_machines group	10

6.2.1 Example of a valid virtual_machine configuration	11

7. Bare-metal application development	12

7.1 Timer	12

7.2 Inter-VM Communication	12

7.3 Unix-like API	13

7.4 UART access	14

7.5 Virtualized I/O	14

## 1. Introduction

This document is intended to show how to configure your host environment and deploy the prplHypervisor with virtualized bare-metal applications to the Microchip PIC32MZ processor and supported boards. The table below shows the supported boards.

| Microchip PIC32MZ Embedded Connectivity Starter Kit | Digilent PIC32MZ Chipkit Wi-Fire | Microchip PIC32MZ Curiosity Board |
| --- | --- | --- |

Current PI32MZ supported boards.

## 2. Prerequisites

- Ubuntu 14.04 LTS or higher. It should work on Debian as well.

- A PIC32MZ supported board.

- Cables for the corresponding board.

## 3. Linux Host Packages Requirements

Perform the following commands to install the required packages on Ubuntu 14.04 LTS.

sudo dpkg --add-architecture i386 sudo apt-get update sudo apt-get install gcc-multilib sudo apt-get install libc6:i386 libncurses5:i386 libstdc++6:i386 sudo apt-get install libexpat1-dev:i386 sudo apt-get install libX11-dev:i386 sudo apt-get install libXext-dev:i386

### 3.1 Remove ModemManager Software Package

The ModemManager software package scans for modems on individual serial ports when they are plugged, e.g. ttyACMx ports. This keeps the port busy for several seconds. To avoid this inconvenience, uninstall it from your system:

sudo apt-get purge modemmanager

## 4. Third Party Software and Tools

- MPLABX IPE already installed (used to flash the bootloader for the first time). You don't need the MPLABX IDE installed, just the IPE;

Installation Instructions: http://microchip.wikidot.com/ipe:installation

- MIPS-MTI toolchain already installed;

Download page: https://community.imgtec.com/developers/mips/tools/codescape-mips-sdk/download-codescape-mips-sdk-essentials/;

Note: Do not forget to add the toolchain to your system PATH variable.

- git client installed;

sudo apt-get install git

- Installed srecord software package;

sudo apt-get install srecord

- libconfig used to read the configuration files;

sudo apt-get install libconfig-dev

## 5. Hypervisor Deployment

This section shows how to obtain the prplHypervisor and deploy it on a board.

### 5.1 Obtaining the prplHypervisor sources

Clone the git repository of the prplHypervisor and make sure you are at the master branch:

git clone https://github.com/prplfoundation/prpl-hypervisor git checkout master

### 5.2 Flashing the bootloader

Use the MPLABX IPE to flash the corresponding bootloader to your board. The bootloader can be found in the prplHypervisor/bin directory. The available bootloaders are:

- Microchip_Curiosity_UART.hex (Microchip PIC32MZ Curiosity board);

- Microchip_Eth_Starter_Kit_UART.hex (Microchip PIC32MZ Embedded Connectivity Starter Kit);

- chipKIT-WiFire-EF.hex (Digilent PIC32MZ Chipkit Wi-Fire Board).

These bootloaders were generated from the project https://github.com/chipKIT32/PIC32-avrdude-bootloader.

Connect the board to your host computer using a standard A to mini B cable. Use the mini B debugger connector on the board. After flashed, the LED1 will keep blinking showing the that the bootloader is ready to upload the firmware.

### 5.3 Compiling the prplHypervisor

From the directory prplHypervisor created by the git clone on step 5, go to the platform directory platform corresponding to your board. The available platform (directories) are:

- platform/pic32mz_starter_kit;

- platform/pic32mz_chipkit_Wifire, and;

- platform/pic32mz_curiosity.

On the chosen directory, use the make command to compile the hypervisor with its default configuration using the MIPS-MTI toolchain. This process will generate the firmware.hex file. This file contains the hypervisor and guests. Example:

~/prplHypervisor/platform/pic32mz_starter_kit$: make gcc -o genconf cfg_reader/genconf.c -lconfig #execute first and exit in case of errors ./genconf samples_cfg/sample-1VM.cfg ¦¦ (exit 1) blink #execute and export to a makefile variable the ouput of the script mips-mti-elf-gcc -EL -O2 -mtune=m14k -mips32r2 -Wa,-mvirt -mno-check-zero-division -msoft-float -fshort-double -c -ffreestanding -nostdlib -fomit-frame-pointer -G 0 -DETHERNET_SUPPORT -DUSB_SUPPORT -DCPU_SPEED=200000000 -DHYPVERSION="v0.11.27 (g9100f1a)" -I../../arch/mips/pic32mz_starter_kit/include -I../../arch/mips/common/include -I../../sys/lib/include -I../../sys/kernel/include -I../../platform/pic32mz_starter_kit/include \ ../../sys/lib/libc.c \ ../../sys/lib/linkedlist.c \ ../../sys/lib/malloc.c ... mips-mti-elf-size blink.elf text data bss dec hex filename 12248 16 1424 13688 3578 blink.elf make[1]: Saindo do diretório `/home/moratelli/hyper/prpl-hypervisor/bare-metal-apps/apps/blink' ../../scripts/genhex.sh blink 0+1 registros de entrada 1+0 registros de saída 65536 bytes (66 kB) copiados, 0,000635281 s, 103 MB/s 0+1 registros de entrada 1+0 registros de saída 32768 bytes (33 kB) copiados, 0,000245427 s, 134 MB/s

Note: See the main Makefile in your platform directory. You may want to modify the input configuration file (CFG_FILE), CROSS_COMPILER or the optional features. Example:

#Input CFG file for VMs configuration CFG_FILE = ../samples_cfg/sample-1VM.cfg # CROSS Compiler CROSS_COMPILER = mips-mti-elf- # Optional Device Drivers CONFIG_INTERVMCOMM = yes CONFIG_PIC32MZ_USB = yes CONFIG_PIC32MZ_ETHERNET = yes CONFIG_VIRTUAL_IO = yes

### 5.4 Uploading the firmware

Upload the resulting firmware.hex file to the board using the make load command. Make sure that you have a cable connected to the UART port on your board. The following table shows the required cable to the supported boards.

| Board | Required Cables | Connections |
| --- | --- | --- |
| Microchip PIC32MZ Connectivity Starter Kit | 2 x Standard A to mini B cables. | Connect the cables to connectors J3 (power) and J11 (UART). |
| Digilent PIC32MZ Chiptkit Wi-Fire | 1 x Standard A to mini B cables. | Connect the cable to connector J1 (power/UART). |
| Microchip PIC32MZ Curiosity board | 1 x Standard A to mini B cables. 1 x Serial to USB adapter. | Connect the mini B cable connector to J3 (power). Use the USB adapter on the UART pins on MikroBus 1. |

Expected make load command output.

~/prplHypervisor/platform/pic32mz_starter_kit$: make load stty 115200 raw cs8 -hupcl -parenb -crtscts clocal cread ignpar ignbrk -ixon -ixoff -ixany -brkint -icrnl -imaxbel -opost -onlcr -isig -icanon -iexten -echo -echoe -echok -echoctl -echoke -F /dev/ttyACM0 ./pic32prog -S -d /dev/ttyACM0 firmware.hex Programmer for Microchip PIC32 microcontrollers, Version 2.0.186 Copyright: (C) 2011-2015 Serge Vakulenko Adapter: STK500v2 Bootloader Program area: 1d000000-1d1fffff Processor: Bootloader Flash memory: 2048 kbytes Boot memory: 80 kbytes Data: 131072 bytes Erase: done Program flash: ################################# done Program rate: 6600 bytes per second

During power on, the bootloader will wait for 3 seconds before automatically loading the application. During this time, it is possible to flash a new firmware. Thus, before try to upload the firmware, press the reset button on your board and wait the blinking led.

### 5.5 Serial output

You can make a cat command to the corresponding serial port see the application’s output. The expected output the default hypervisor configuration is listed below. The default configuration generates the ping-pong application.

=========================================================== prplHypervsior v0.11.112 (ge35a934) [Oct 13 2016, 20:26:29] Copyright (c) 2016, prpl Foundation =========================================================== CPU ID: M5150 ARCH: Microchip Starter Kit SYSCLK: 200MHz Heap Size: 29Kbytes Scheduler 5ms VMs: 2 Initializing Virtual Machines. Configuring ping VM starting at 0x80008000 RAM address. Configuring pong VM starting at 0x80010000 RAM address. PIC32mz in Vectored Interrupt Mode. Inter-VM communication hypercalls registered. Software reset interrupt (SW1) registered at offset 0x220. Initializing USB device in Host mode. USB interrupt vector at 0x240 Device mapping hypercalls registered. Ethernet PHY at 0 Ethernet interface en0: interrupt 153, MAC address d8:80:39:75:a6:1a Ethernet device is a SMSC LAN8740A CP0 Timer interrupt registered at 0x260. ping VM ID 1 Target VCPU not pong VM ID 2 initialized. pong VM: message from VM ID 1: "ping? 5" (8 bytes) ping VM: message from VM ID 2: "pong!" (6 bytes) pong VM: message from VM ID 1: "ping? 20" (9 bytes) ping VM: message from VM ID 2: "pong!" (6 bytes) pong VM: message from VM ID 1: "ping? 35" (9 bytes) ping VM: message from VM ID 2: "pong!" (6 bytes) pong VM: message from VM ID 1: "ping? 50" (9 bytes) ping VM: message from VM ID 2: "pong!" (6 bytes) pong VM: message from VM ID 1: "ping? 65" (9 bytes) ...

## 6. Hypervisor configuration

The prplHypervisor configuration is made through readable configuration files (cfg). During the compilation, the cfg file is read and interpreted to generate the config.h header in the platform directory. The prplHypervisor/platform/cfg directory has some configuration file samples. The cfg files are structured in two main groups: system and virtual_machines. The system group contains system wide configuration, while the virtual_machines group contains the configuration of each VM in the resulting system. The properties of each group are detailed below.

### 6.1 system group

System group properties description.

| Propriety | Type | Description | Valid Range |
| --- | --- | --- | --- |
| debug | Array of strings | Optional list of defines for output messages for debug purposes. | WARNINGS, INFOS, and ERRORS. |
| platform | Proprierty group | Specific platform information | Not applicable |
| platform.cpu | string | CPU description. | Any valid string |
| platform.platform_str | string | Platform description | Any valid string. |
| platform.system_clock | integer | Processor's clock in Hertz. | Positive integers |
| uart_speed | integer | Console UART baudrate | 115200, 57600 and 9600. Other baud rates may be used. |
| scheduler_quantum_ms | integer | Timer interval in millisecods for the hypervisor scheduler timer (QUANTUM) | Positive integers. |

#### 6.1.1 Example of a valid system configuration

system = { debug = [ "WARNINGS", "INFOS", "ERRORS"]; platform = { cpu = "M5150"; platform_str = "Microchip Starter Kit"; system_clock = 200000000; }; uart_speed = 115200; scheduler_quantum_ms = 5; };

### 6.2 virtual_machines group

Description of the virtual_machines group properties.

| Property | Type | Description | Valid Range |
| --- | --- | --- | --- |
| app_name | string | Name of the bare-metal application directory. | Any valid bare-metal app directory name. |
| os_type | string | Type of the virtualized OS. | BARE_METAL |
| RAM_size_bytes | string | Size of the VM's SRAM. | MEM_SIZE_32KB MEM_SIZE_64KB MEM_SIZE_128KB MEM_SIZE_256KB |
| flash_size_bytes | string | Size of the VM's FLASH. | MEM_SIZE_32KB MEM_SIZE_64KB MEM_SIZE_128KB MEM_SIZE_256KB MEM_SIZE_512KB MEM_SIZE_1MB |
| fast_interrupts | Array of strings | List of associated interrupts. Allows for the hypervisor to fast delivery an interrupt to a VM. | IRQ_ETH |
| device_mapping | Array of strings | List of associated devices for virtualized I/O. | PORTA through PORTK, UART1 through UART6, SPI1 through SP6, PM and AD1. |
| memory_maps | Array of properties | Indicate the VM's memory mapping. | Not applicable. |
| memory_maps.base_addr | integer | Memory mapping base address | Any virtual address |
| memory_maps.page_size | string | Memory mapping size | MEM_SIZE_4KB MEM_SIZE_8KB MEM_SIZE_16KB MEM_SIZE_32KB MEM_SIZE_64KB MEM_SIZE_128KB MEM_SIZE_256KB MEM_SIZE_512KB MEM_SIZE_1MB |

#### 6.2.1 Example of a valid virtual_machine configuration

The blink VM has 32KB of SRAM, 64KB of FLASH access to PORTH and UART2 through virtualized I/O. The pong VM has 32KB of SRAM and FLASH, access to UART2 through virtualized I/O and a 4KB page mapped for direct peripheral access on base address 0xbf860000.

virtual_machines = ( { app_name = "blink"; os_type = "BARE_METAL"; RAM_size_bytes = "MEM_SIZE_32KB"; flash_size_bytes = "MEM_SIZE_64KB"; device_mapping = [ "PORTH", "UART2"]; }, { app_name = "pong"; os_type = "BARE_METAL"; RAM_size_bytes = "MEM_SIZE_32KB"; flash_size_bytes = "MEM_SIZE_32KB"; device_mapping = [ "UART2"]; memory_maps = ({ base_addr = 0xbf860000; page_size = "MEM_SIZE_4KB"; }) } );

## 7. Bare-metal application development

To develop new applications move to the ~/prplHypervisor/bare-metal-apps/apps directory and see the blink, ping, pong directories among others examples. Each directory contains the implementation of a bare-metal application. These applications became guests during the hypervisor execution.

The hypervisor can perform up to 6 bare-metal applications (virtual machines). Each virtual machine has configurable SRAM and FLASH sizes, as explained above. To compile different bare-metal applications, modify the input configuration file in platform/<platform_directory>/Makefile, as in the following example:

#Input CFG file for VMs configuration CFG_FILE = cfg/sample-1VM.cfg

The easiest way to implement a new bare-metal app is to copy the directory of an existing application, for example blink, to a new directory and modify the implementation. Additionally, to compile different combinations of applications see the explanation in Section 6.

### 7.1 Timer

The hypervisor scheduler quantum can be configured in scheduler_quantum_ms property (see section 6.1). Thus, each VM will perform for scheduler_quantum_ms milliseconds before it is preempted. During its execution, the guest will receive timer interrupts each millisecond. To handle the timer interrupts, the guest must register an interrupt handler calling:

interrupt_register(irq_timer, GUEST_TIMER_INT);

Where irq_timer is the interrupt handler.

### 7.2 Inter-VM Communication

The prplHypervisor supports Inter-VM communication through message copies from origin to the target VM. The API is simple and consists of the following calls:

int32_t ReceiveMessage(uint32_t *source, char* buffer, uint32_t bufsz, uint32_t block); uint32_t SendMessage(uint32_t target, char* buffer, uint32_t size);

Each VM has an identification number (ID) generated by the hypervisor during its initialization. The ID is attributed following the VM's initialization order. The first initialized VM’s ID is 1, the second is 2 and so on. The ID is used to identify the VMs during communication. A VM can discover its own ID calling the hypercall get_guestid().

The ReiceveMessage() hypercall returns greater than 0 if a message was received. The data is copied to the buffer limited to bufsz bytes. Zero means no data was received.  The SendMessage() hypercall returns the number of bytes sent. Zero or less means an error. The follow table shows the possible error codes.

| Error Code | Meaning |
| --- | --- |
| MESSAGE_VCPU_NOT_FOUND | Destination not found. |
| MESSAGE_FULL | Destination queue full. |
| MESSAGE_TOO_BIG | Message to big. |
| MESSAGE_VCPU_NOT_INIT | The target VPCU is not ready for communication. |

If non-blocking receive is desired, use block equal to zero when calling ReceiveMessage(), otherwise, it will block the application until a message is received.

The application Ping-Pong is an example of inter-VM communication. A message is sent from ping (VM ID 1) to pong (VM ID 2) and back. A cfg file is provided with a configuration sample for inter-VM communication. The cfg file can be found at prplHypervisor/platform/cfg/sample_2VMs.cfg. To compile the Ping-Pong example, it is needed to adjust the CFG_FILE variable in the platform Makefile to:

#Input CFG file for VMs configuration CFG_FILE = ../samples_cfg/sample-2VMs.cfg

### 7.3 Unix-like API

The bare-metal application's execution environment supports a minimal Unix-like API. The available functions are listed below:

int8_t *strcpy(int8_t *dst, const int8_t *src); int8_t *strncpy(int8_t *s1, int8_t *s2, int32_t n); int8_t *strcat(int8_t *dst, const int8_t *src); int8_t *strncat(int8_t *s1, int8_t *s2, int32_t n); int32_t strcmp(const int8_t *s1, const int8_t *s2); int32_t strncmp(int8_t *s1, int8_t *s2, int32_t n); int8_t *strstr(const int8_t *string, const int8_t *find); int32_t strlen(const int8_t *s); int8_t *strchr(const int8_t *s, int32_t c); int8_t *strpbrk(int8_t *str, int8_t *set); int8_t *strsep(int8_t **pp, int8_t *delim); int8_t *strtok(int8_t *s, const int8_t *delim); void *memcpy(void *dst, const void *src, uint32_t n); void *memmove(void *dst, const void *src, uint32_t n); int32_t memcmp(const void *cs, const void *ct, uint32_t n); void *memset(void *s, int32_t c, uint32_t n); int32_t strtol(const int8_t *s, int8_t **end, int32_t base); int32_t atoi(const int8_t *s); int8_t *itoa(int32_t i, int8_t *s, int32_t base); int32_t puts(const int8_t *str); int8_t *gets(int8_t *s); int32_t abs(int32_t n); int32_t random(void); void srand(uint32_t seed); int printf(const int8_t *fmt, ...); int sprintf(int8_t *out, const int8_t *fmt, ...); void udelay(uint32_t usec); void putchar(int32_t value); uint32_t getchar(void);

### 7.4 UART access

Some platforms may have a secondary UART port. To select between the available UART ports use the following call:

int32_t serial_select(uint32_t serial_number);

Verify the available UART ports on your platform on the following file:

prplHypervisor/bare-metal-app/platform/<specific platform directory>/include/uart.h

### 7.5 Virtualized I/O

For a complete guest memory space separation use the property device_mapping to describe which peripherals the guest can access through virtualized I/O. On the guest, use read()/write() calls for peripheral access.

For example:

uint32_t v; write(TRISHCLR, 1); v = read(LATD);